Reviewer1#451C

a. Proof Process of Differences in Signal Quality

To illustrate the Differences in Signal Quality (DSQ) phenomenon, we analyzed the results of the CNN model1 on simulated data with a signal-to-noise ratio (SNR) of 15. The analysis focused on three types of signals: the clean signal Cleansn, the noisy signal Noisysn, and the predicted signal Predsn. Based on these signals, three metrics were calculated to evaluate the quality of individual signals: Noise, Fitting Error, and Reconstruction ErrorRecon Error.

Because the closeness of a signal to Cleansn serves as an indicator of its quality, Noise can be used to evaluate the quality of Noisysn. Recon Error can be used to evaluate the quality of Predsn. It should be noted that smaller values of Noise and Recon Error indicate that the corresponding Noisysnor Predsn is closer to Cleansn, and therefore has higher quality.

These metrics can be calculated as follows:

Noise=Noisysn - Cleansn2# seq eq 2

FittingError=Predsn - Noisysn2# seq eq 3

ReconError=Predsn - Cleansn2# seq eq 4

In the CNN1 simulation experiment, we calculated the mean Noise and mean Recon Error of all signals and plotted their variations with training epochs, as shown in Fig. 1. Noise refers to the difference between the original noisy signals and the clean signals; its mean value remains constant across epochs and therefore appears as a horizontal line in the figure. Recon Error, in contrast, denotes the difference between the predicted signals and the clean signals. As the number of epochs increases, the predicted signals are continuously updated.  Therefore, the mean Recon Error varies with the Epoch.

At epoch 1, the mean Recon Error of the model is higher than the mean Noise. Because smaller values of Noise and Recon Error indicate that the corresponding Predsn and Noisysn are closer to Cleansn, reflecting higher overall signal quality. Therefore, at epoch 1, the overall quality of Predsn is lower than that of Noisysn. This can be attributed to the fact that the model is still in the early training stage and has not yet effectively learned the signal features, resulting in a lower quality of Predsn. As training progresses, the mean Recon Error gradually decreases, indicating that the overall quality of Predsn improves and eventually surpasses that of Noisysn in later epochs. This phenomenon can be summarized as Phenomenon 1: As training progresses, the predicted signals progressively surpass the noisy signals in overall quality.	​

Figure 1: Mean Noise (red) and mean Recon Error(blue) of all signals across training epochs, computed according to Eq. 2 and Eq. 4. After epoch 10, Recon Error becomes stable. Therefore, metrics are reported every 20 epochs starting from epoch 10. Data are derived from the CNN1 simulation experiment with SNR = 15.

To further investigate the signal characteristics within a single epoch, a refined analysis was conducted based on Fig. 1. Specifically, all signals within one epoch were grouped according to their Fitting Error, which was calculated as Eq. 3. It should be noted that while the overall mean Noise across all signals is fixed, the mean Noise within each subgroup varies due to differences in the sampled signals. To enhance the reliability of the conclusions, the relationships among Noise, Recon Error, and Fitting Error were examined across multiple epochs (30, 50, 100). The results are presented in Fig. 2.

From Fig. 2, another phenomenon can be observed: as the Fitting Error increases, the mean Noise increases significantly, while the mean Recon Error decreases slightly. This phenomenon can be observed for all epochs greater than 10. Because smaller values of Noise and Recon Error indicate signals closer to the clean signal, reflecting higher overall quality, this phenomenon can be summarized as Phenomenon 2: After extended training, as the Fitting Error increases, the overall quality gap between predicted signals and noisy signals increases.

Figure 2: Mean Noise (dashed) and mean Recon Error (solid) of signals grouped by Fitting Error within a single epoch. Fitting Error intervals are left-closed, right-open. Data from epochs 30, 50, and 100 are shown in red, blue, and green curves, respectively. Data are derived from the CNN1 simulation experiment with SNR = 15.

The verification process for Phenomena 1 and 2 is as follows:

Consider n clean signals across all voxels and all b-values, with the normalized clean signals denoted as si and the noise introduced during acquisition denoted as ϵi. The noisy signal is represented as ni=si+ϵi. The predicted signal obtained from an unsupervised model using ni as input is denoted as pi , and the corresponding fitting error is defined as fi=pi-ni.

To reflect the overall quality of the predicted signals, we define the metric Qp​ as follows:

Qp=1ni=1npi-si# seq eq 5

Since Qp represents the differences between the predicted signals piand the clean signals si, a smaller value of Qp indicates that pi is closer to si, reflecting higher overall signal quality. Similarly, we define Qn to quantify the overall quality of the noisy signals ni:

Qn=1ni=1nni-si# seq eq 6

From the definitions of fi and ni, we have pi = ni + fi and ni- si=ϵi, which leads to:

pi-si=ni+fi-si=ni-si+fi=ϵi+fi# seq eq 7

Qp=1ni=1npi-si=1ni=1nϵi+fi# seq eq 8

Qn=1ni=1nni-si =1ni=1nϵi# seq eq 9

The study by Zhang et al2 demonstrated that, after extended training, the error in the loss function is primarily attributable to noise. To reduce the loss, the model tends to learn the noise ϵi and suppress its influence, thereby driving the predicted signal closer to the clean signal. Since pi=ni+fi, the fitting error fi can be regarded as the correction made by the model when processing the noisy signal ni. Consequently, after sufficient training, for the vast majority of voxels and b-value signals, fi is oriented in the opposite direction of ϵi, with its magnitude being smaller than that of ϵi, thus:

ϵi+fi<ϵi# seq eq 10

Therefore:

pi-si=ϵi+fi<ϵi=ni-si# seq eq 11

On average over all signals, we have:

Qp​=1ni=1npi-si<1ni=1nϵi=Qn​# seq eq 12

Smaller values of Qp and Qn indicate that the corresponding predicted signals pi and noisy signals ni exhibit higher overall quality. This finding strongly supports Phenomenon 1: As training progresses, the predicted signals progressively surpass the noisy signals in overall quality.

The quality metric of a single predicted signal is defined as qp = pi-si, while the quality metric of a single noisy signal is defined as qn = ni-si. The quality difference between the two is represented as ∆i=qp-qn.

Smaller values of qp and qn indicates that the corresponding predicted signal pi or noisy signal ni is closer to the clean signal si, thereby reflecting higher quality. Therefore, if ∆i<0, the predicted signal pi has higher quality than the noisy signal ni; if ∆i>0, the predicted signal pi has lower quality than the noisy signal ni, with larger values of ∆i indicating a greater quality difference between pi and ni.

Expanding ∆i as：

∆i =qp-qn =pi-si - ni-si# seq eq 13

Substituting Eq. 11 into Eq. 13:

∆i=ϵi+fi-ϵi# seq eq 14

Discussion by cases:

∆i=-fi, ϵi+fi<ϵi   fi, ϵi+fi>ϵi## seq eq 15

From Eq. 10, it follows that after extended training, for the vast majority of voxels and b-value signals, ϵi+fi<ϵi, in this case ∆i=-fi. That means the larger the absolute value of the fitting error fi, the larger |∆i| and the smaller ∆i, indicating a more significant quality improvement of the predicted signal pi relative to the noisy signal ni.

Since this relationship holds for the vast majority of voxels and b-value signals, the predicted signals exhibit higher overall quality than the noisy signals across all voxels and b-values. Moreover, the larger the absolute value of the fitting error fi, the more pronounced this advantage becomes. This finding strongly supports Phenomenon 2: After extended training, as the Fitting Error increases, the overall quality gap between predicted signals and noisy signals increases. It should be noted that, in the actual statistical analysis, the fitting error is assumed to be positive by default; therefore, fitting error in Phenomenon 2 refers to fi rather than fi.

b. Related Experimental Logs

Revierer2#4F5E

- convergence plot

Figure 3: RMSE of SI-CNN in predicting F, D, and D* parameters (computed as the

mean squared error between the predicted parameters and the ground truth) and RMSE

of training signals (computed as the mean squared error between the training signals

and the clean signals) as a function of the times of iterations for simulated abdominal

data at SNR = 15. The x-axis indicates the times of data iterations.。

Figure 4：Training and Validation Loss.

Reference

Huang, H. M. (2022). An unsupervised convolutional neural network method or estimation of intravoxel incoherent motion parameters. Physics in Medicine & Biology, 67(21), 215018.

Zhang, K., Zuo, W., Chen, Y., Meng, D., & Zhang, L. (2017). Beyond a gaussian denoiser: Residual learning of deep cnn for image denoising. IEEE transactions on image processing, 26(7), 3142-3155.